On the Result of Underground Temperature Survey for Detecting the Groundwater Vein-stream at the Barikan Landslide Area in I. R. Iran

Gen Furuya, Atsuo Takeuchi, Zieaodin Shoaei, Jafar Ghayoumian

Disaster Prevention Research Institute

Kyoto University, Uji, Kyoto, Japan

E-mail: furuya@landslide.dpri.kyoto-u.ac.jp

Abstract

It is recognised that rainwater infiltrating into soil tends to move in a vertical direction, and reduce its velocity and decreases horizontally when it reaches an impermeable layer. It is necessary to determine exactly the location and depth of the groundwater vein-stream flowing above the impermeable layer for effective use of the groundwater in arid climate area.

In order to accomplish this purpose, an electric resistance survey and electric logging were carried out primarily for obtaining this information. Using these methods it is very difficult to obtain information on the location and depth of the groundwater vein-stream.

An underground temperature survey method has been developed to detect accurate information on the groundwater vein-stream in Japan. This method utilises the different temperatures of the groundwater vein-stream, as well as ground temperature of the shallow layer. It is composed of the ground temperature survey at a depth of one meter, multiple points temperature logging, usual temperature logging, and measurement of direction and the velocity of groundwater vein-stream.

The groundwater investigation method, using temperature is extensively used in Japan as a useful method for research carried out on groundwater vein-stream, underground flow, groundwater disaster, groundwater obstruction and groundwater resources.

However the principle of the method is generally based on the following assumption. Since more frequently a green rich oasis in barren deserts in arid climate area or areas having very poor vegetal cover are formed around springs, which are the outcrop of the groundwater vein-stream, it forms sites with different temperatures in comparison to others. In these areas, walking with bare feet during the summer and fall seasons, particularly during the day, is impossible. At the same time it is possible to walk barefoot around spring outlets and neighbouring sites. The most basic principle of the investigation method of the groundwater vein-stream by measuring underground temperature is the same as these phenomena.

Based on this fact, it is suggested that one detects the location of the groundwater vein-stream by measuring the ground temperature at shallow layers of the soil. Therefore, using this method it is possible to measure temperature distribution of the soil surface, in areas with harsh and abnormal conditions such as the deserts or the Sahara. This in mostly because if there is groundwater vein-stream, the ground temperature around it will be lower because of the effect of the existence of the vein, particularly in the summer and autumn. This is due to the lower temperature of the ground around the vein-stream in comparison to the ground temperature where there is no vein-stream. On the other hand, the ground temperature of the vein-stream is obviously higher than other sites in winter and spring.